# Histogram Exam Notes

Vương Quốc Trung  |  Group 03 · Exam revision · A4 duplex

Speech and silence segmentation. Review the features, threshold learning, inference and evaluation used in the submitted program.

# 1 Data and framing

TRAIN contains phone_F1, phone_M1, studio_F1, studio_M1; TEST uses the corresponding F2 and M2 files. Existing LAB rows are start_s end_s label: sil → silence/background; voiced v and unvoiced uv → Speech. Use the label at each decision-cell center. LAB does not detect speech itself.

A 25 ms window measures local energy; a 10 ms hop tracks changes and locates boundaries on a time grid. Windows overlap; the first center is 5 ms. At WAV edges, use only real samples without zero padding.

L = round(0.025 fs);   H = round(0.010 fs)

fₛ is the sample rate in Hz; L is the window length in samples; H is the hop in samples. At 16 kHz: L=400, H=160; at 44.1 kHz: L=1102, H=441 with Python round.

# 2 STE and normalization per WAV

Ek = n ∈ Wk
                x[n]2Nk;   zk = Ekmaxj Ej

x[n] is an int16 sample divided by 32768; Wₖ is window k; Nₖ is its actual sample count. Eₖ is raw mean-square STE; zₖ is dimensionless normalized STE in [0,1]. Compute all E values, find this WAV’s maximum, then divide each E by it. If the maximum is zero, return z=0 and retain Silence.

MAk = n ∈ Wk
                |x[n]|Nk;   logSTE = 10 log10(max(E, 10−12))

MA is mean absolute amplitude; logMA=20 log₁₀(max(MA,10⁻¹²)). MA/log features illustrate results; all three segmentation thresholds use normalized STE.

# 3 From histogram to peaks and threshold

B = 128;   Δz = 1B;   cj = j + 0.5B;   j = 0, …, 127

Divide [0,1] into B bins of width Δz; cⱼ is bin j’s center. hⱼ counts zₖ values in that bin; the final bin includes 1. Smooth h with a three-bin moving mean, repeating edge values. The horizontal axis is normalized STE and the vertical axis is frame count, not time.

h̃j = hj−1 + hj + hj+13;   h−1 = h0;   h128 = h127

A peak is a positive local maximum above both neighbors. Merge an equal-height plateau into one peak: use index 0 if it starts at bin 0; otherwise use the floored middle index. Code chooses the first two peaks along the STE axis, not the two tallest peaks; bin 0 is eligible.

T = W cp1 + cp2W + 1

p₁<p₂ are the selected peak indices; W>0 weights the first peak’s center. Larger W moves T toward cₚ₁, often retaining lower-energy Speech but potentially increasing false alarms. Peak locations, not heights, determine T. With fewer than two peaks, use this WAV’s mean(z).

Current code has no valley-depth, minimum-peak-distance or lowest-20%-region condition. The two peaks can therefore both belong to Speech or arise from noise; this is a genuine limitation of the selection rule.

# 4 Learn W on TRAIN

Try W ∈ {1,2,5,10,15,20,30,50}. For each W, compute a histogram and T separately for each TRAIN file, segment, then compare with LAB. Rank by fewest extra+missed boundaries, lowest mean MAE, then smallest W. Select W=30, TRAIN MAE=12.5 ms, no extra/missed boundaries. B=128 and smoothing order 3 remain fixed.

# TEST example and thresholds

At TEST time, freeze W and derive T from the current WAV’s histogram without using LAB. Keep 25/10 ms framing and the 200 ms Silence filter. Histogram does not median-filter the STE sequence; smoothing a histogram differs from temporal median filtering.

phone_F2: p₁=0, p₂=6 → c₁=.00390625, c₂=.05078125. T=(30×.00390625+.05078125)/31=.005418346774. Smoothed peak heights of about 171 and 6.67 identify peaks but do not enter the threshold formula.

# 5 TEST and evaluation

TEST → frames → STE → per-WAV normalization → dₖ=zₖ. dₖ≥T → Speech; below T → Silence. Fill Silence <200 ms, including edge intervals; retain ≥200 ms. TEST LAB is only for evaluation and reference plots.

ei = 1000(tpred,i − tref,i);   MAE = i=1M|ei|M;   RMSE = i=1Mei2M

t is in seconds; e, MAE and RMSE are in ms; M is the number of matched boundary pairs. Greedily match nearest boundaries of the same direction, once each, with no cutoff. Extra/missed boundaries are excluded from MAE and must be reported separately; no pairs means None, not zero.

| TEST | Peak bins | T | MAE ms | RMSE ms |
| --- | --- | --- | --- | --- |
| phone_F2 | 0, 6 | .005418347 | 10 | 10 |
| phone_M2 | 0, 9 | .006174395 | 5 | 7.07 |
| studio_F2 | 0, 9 | .006174395 | 15 | 15.81 |
| studio_M2 | 0, 4 | .004914315 | 15 | 15.81 |
| Mean per file |  |  | 11.25 | 12.17 |

All three algorithms: 8 matched pairs, 0 extra, 0 missed. Mean per file averages four metrics and is not pooled RMSE. Gaussian has the lowest mean MAE on these four TEST files; this does not establish superiority on all data.

BER = rFA + rMiss2;   rFA = n(Sil → Sp)n(Sil);   rMiss = n(Sp → Sil)n(Sp)

n counts frames with valid reference labels; evaluate after postprocessing. rFA is the Silence-to-Speech rate; rMiss is the Speech-to-Silence rate. Multiply BER by 100 for %. This classification error differs from energy errors and boundary MAE.

# Diagnostic plots and common traps

Bars show raw counts; the smooth curve shows the three-bin mean; marked points are the first two peaks; the vertical line is T. LAB does not build the histogram. TRAIN LAB selects W, so the full procedure is not completely unsupervised. W=30 does not mean 30-bin smoothing; smoothing uses only 3 bins.

# 6 F0 and SNR if asked

F0 = fsℓ*;   SNR = 10 log10(PSp − PSilPSil)

F0 (Hz): fundamental frequency; ℓ*: autocorrelation peak lag after DC removal and Hamming windowing. Accept 60–400 Hz with z≥0.01 and normalized peak ≥0.30. F0 does not control segmentation. SNR (dB) uses LAB-region powers PSp and PSil, assuming comparable noise; invalid ratios return None.

Code: features.extract; TT2_Histogram.smooth_1d, histogram_local_maxima, histogram_threshold; pipeline.choose, predict; evaluation.boundary_scores, score.